The NETWORK LAYER

Overview

- Routing Protocols

- IP protocol

- ICMP protocol

Internet Protocol (IP)

IP Packet Structure

- Version Number:

  - Indicates version of IP protocol

  - Typically, ‘4’ for IPv4

- Header Length:

  - Number of 32-bit words in header

  - Typically, ‘5’ for a 20 byte IPv4 header

  - Can be more when IP options used

- Total Length:

  - Total number of bytes in the packet

- 8-bit Protocol:

  - Identifies higher level Transport protocol (TCP/UDP)

  - Important for demultiplexing at receiving host

    - It’ll know whether to continue to send the packet on TCP or UDP

- Time to Live (TTL):

  - Actually indicates the max number of counts

  - If you set TTL to 4, the 5th router would actually drop the packet

Preventing Router Loops using TTL

- Problem:

  - Forwarding loops cause packets to cycle and eventually consume all capacity

- TTL can be set so that once the TTL limit is reached, it’ll drop out and stop the loop

Preventing Header Corruption using Checksum

- Checksum only applies to the header for IP and NOT the payload

  - This is because TCP already covered the payload with its checksum, so IP doesn’t have to

- IP checksum recalculated at each router

  - This is because TTL changes between routers so the checksum needs to be changed too

  - If the checksum addition is wrong then the packet is discarded

IP Fragmentation

- Network links have a Maximum Transfer Size (MTU):

  - Maximum frame a link can carry

  - Different types of links have different MTUs

- A Large IP datagram has to be fragmented if its bigger than the MTU

  - Reassembled at final destination (NEVER AT THE ROUTER)

  - IP header bits are used to identify and order related fragments

- ID Flag:

  - ID of the original datagram

  - All subsequent fragments if the datagram would share the same ID

- MF Flag:

  - More Fragments flag

  - All fragments except the last will have 1

  - Last will be 0 because there are no more fragments

- Offset:

  - If offset is 0 and MF is 1, it means it is the FIRST fragment

  - Offset size is actual data (1480) / 8

- Length:

  - Data is really split into MTU – 20 (header size)

  - So 4000 = 3980 bytes

    - F1 = 1480

    - F2 = 1480

    - F3 = 1020

Receiving host uses MF, Offset, and ID to reassemble the datagram

Malicious use of fragments

- Sending two fragments with ‘overlapped offsets’ which some OS can’t handle and fucks the whole system lmao

Overall Overhead:

- 20 bytes TCP header

- 20 bytes IP header

- App layer overhead

IPv4 Addressing

IP Address:

- 32-bit identifier for host-router interface

- Every device MUST have an IP address

- For devices on the same network, only the last digit tends to be different

Interface:

- Connection between host/router and physical link

  - Routers typically have multiple interfaces

    - Example on right has 3

  - Hosts typically only have 1 or 2 interfaces

Networks!

What’s a network?

- Device interfaces with same network part of the IP address

- Devices in the same network can reach each other without going through the router

Masking

- Used in conjunction with network address to extract the Network part and host part of the address

- Bitwise AND operator used to produce network part and Host part

Classless InterDomain Routing (CIDR)

- Used in todays addressing

- Subnet addresses are of arbitrary length

- Address format: a.b.c.d/x

  - where x is # bits in subnet portion of addresses

Receiving IP Addresses

Dynamic Host Configuration Protocol (DHCP)

- Allows host to dynamically obtain its IP address from network server when it joins the network

- Allows reuse of addresses

  - Only holds address while device is connected

- Overview of DHCP

  - Host/device broadcasts ‘DHCP discover’ message

  - DHCP server responds with ‘DHCP offer’ message

  - Host requests IP with ‘DHCP request’ message

  - DHCP server gives address: ‘DHCP Ack’ message

Range of addresses DHCP servers can assign

- If theres a subnet 129.94.100.0/25

  - We cannot use the network and broadcasting addresses:

    - 129.94.100.0 && 129.94.100.127

  - Therefore, DHCP can only use 129.94.100.1  129.94.100.126

NETWORK ADDRESS TRANSLATION (NAT)

Private IP addresses

- 10.0.0.0/8

- 172.16.0.0/12

- 192.168.0.0/16

Features of private IPs

- Private IP’s cannot be routed

- Typically used for NAT (Network Address Translation)

Routing Protocols: Intra-Domain Routing (Link State / Distance Vector) and Inter-Domain Routing

A routing protocol determines the end-to-end path of packets through the network.

The forwarding table determines the local forwarding at this router.

Autonomous Systems (AS) or Domains is a region of a network under a single admin authority. E.g. an ISP is an AS.

Internet routing works as two levels:

- Intra-Domain Routing Protocol: AS Establishing routes within its own AS / domain.

  - Single admin, so no policy decisions are needed. Performance > Policy.

  - Examples of intra-domain routing:
Link State  Open Shortest Path First (OSPF)
Distance Vector  Routing Information Protocol (RIP)

- Inter-Domain Routing Protocol: AS Establishing routes between other AS / domains

  - Admin wants control over routing in network + who routes through its network. Policy may > Performance.

  - Examples of inter-domain routing:
Path Vector  Border Gateway Protocol (BGP)

Link State Routing (Global): All routers have the complete topology and maintain / know the cost of each link in the network.

- How it works: (1) Link State Advertisement (LSA) Flooding (2) Path calculation with Djikstra’s

  - When receiving a new Link State msg, the router forwards it to all neighbours except one that sent the msg.

  - Routers keep a local copy so they don’t forward previously seen LSA’s.

  - Eventually, each node learns the entire network topology + can use Djikstra’s to compute shortest path.

- Eventually, each node learns entire network.

- Characteristics

  - Connectivity / cost changes are flooded to all routers in the network.

  - Converges quickly (less consistency, looping)

  - Limited network sizes, otherwise it will be too costly.

- Challenges:

  - Packet Loss / Out-of-order packets (solved with ACKs, Retransmissions, Seq Numbers, TTL for packets)

  - Scalability: # Messages to flood O(N*E) where N = #nodes E = #edges  |   Djikstra’s O(N2)
                    # entries in topology database O(E)   |    # entries in forwarding table O(N)

  - Transient Disruptions / Infinite Loop problems: Inconsistent link-state database, as some routers know about failures before others. Shortest path is not always consistent, which can cause transient / infinite loops.

  - Oscillations: Costs can change around continuously. For given new costs  new route  new costs and so on.

Distance Vector Routing (Decentralised): Routers only know its neighbours + link cost to neighbours.

- How it works:
(1) Each router initialises its DV table based on link costs to immediate neighbours + sends its DV to the neighbours.
(2) Neighbours process the DV and repeats STEP #1 until the iterative process converges to a set of shortest paths.
(3) Each node then waits for changes in their local link cost or msg from neighbours.
(4) If change occurs  recompute costs in DV and notify neighbours if anything changes.

- Initial state: best 1-hop paths | one simultaneous round = best 2-hop | k simultaneous rounds = best (K+1)-hop paths

- Characteristics:

  - Cost changes are iterative, exchanges info from neighbour to neighbour.

  - Requires multiple rounds to converge

  - Scales to large networks.

- Counting to Infinity Problem (“bad news travels slowly”): Usually occurs when a node becomes broken.

  - Because of a broken link, nodes keep incorrectly updating their DV table and increasing cost for the broken link until the updates slowly propagates through the network and eventually reaches infinity.

- Poisoned Reverse Rule is a method to avoid the Count to Infinity Problem.

  - Routers actively advertise certain links as unreachable (cost=infinity). However, this will significantly increase the number of routing announcements made in the network.

Comparison of Link State vs. Distance Vector

|  | Link State | Distance Vector |
| --- | --- | --- |
| Message Complexity | N nodes / E edges = O(N*E) messages sent | Exchange between neighbours only. |
| Speed of Convergence | O(N2) algorithm ¦ relatively fast | Convergence time varies Count to Infinity / Routing Loops may occur |
| Robustness | LS node can advertise incorrect LINK cost. Each node computes only its own table. | DV node can advertise incorrect PATH cost. Each node’s table is used by others, errors propagate through the network. |

Problems with Distance Vector:

- Most problems caused by slow convergence of routers or converging on wrong info

- Convergence:

  - The time in which all routers come to an agreement about the best paths in network

  - Reacts rapidly to good news (finding low-cost path) and slowly to bad news (cost increase)

    - Bad news would slowly iterate and can create the ‘count to infinity’ problem

ICMP: Internet Control Message Protocol

- Used for ping and traceroute

- ICMP message codes are sent between routers to give info